ISET de Radès

Département Technologies de l’Informatique

Mini projet : Contrôle d’accès avec reconnaissance faciale

Réalisé par Fedi Sabri de L1I5

Guidé par Mr Sami Melki

2023

# Matériel requis :

| RPi3B | Ordinateur single board, système d’exploitation Raspbian et Connecté à l’internet |  |
| --- | --- | --- |
| Smartphone Android (cam) | Pour camera |  |
| Câbles | Câbles de connexion |  |
| Câble USB vers Micro-USB | Identique aux câbles de du téléphone portable |  |
| Câble USB vers USB type-C | Identique aux câbles de charge du téléphone portable |  |
| Pc pour développement | Système d’exploitation Windows et Connecté à l’internet |  |
| Diodes led | Vert et rouge pour indiquer la réponse |  |
| Bottons |  |  |

# Table des matières

I)Configurer la carte et l’environnement de développement	4

1)	Raspberry pi imager	4

2)	Connecter vs code à travers SSH	8

3)	Installation des bibliothèques et binaires nécessaires	8

4)	Installation de flutter et Dart	9

5)	Installation de Android studio et création de machine virtuelle	10

II)	Le schéma principal	14

III)	Le montage	14

IV)	Le code	15

6)	Le code de la carte	15

7)	Le code coté serveur	18

8)	le code de l’application client	22

9)	Capture d’écran de l’application	35

# Configurer la carte et l’environnement de développement

## Raspberry pi imager

Téléchargez et lancez Raspberry pi imager setup

Suivez l’installateur

Une fois installé, lancez-le.

Cliquez sur « Choose OS »

Configurez les options avancées en appuyant sur les touches Ctrl + Maj + X pour faire apparaitre ce menu (SSH pour pouvoir utiliser la carte en mode headless)

Cliquez sur « Choose SD card » puis sélectionnez votre carte micro SD

Cliquez sur « write »

Cliquez sur « ok »

## Connecter vs code à travers SSH

Installez l’extension « remote ssh » par Microsoft

Tapez “Remote-SSH: Connect to Host” dans la palette des commandes et écrivez l’adresse IP locale de votre carte puis entrez votre mot de passe

Sélectionnez le type du système si demandé

## Installation des bibliothèques et binaires nécessaires

- Sur serveur

py -m pip install deepface

py -m pip install sqlite3

py -m pip install flask

py -m pip install flask_cors

winget install git

- sur la carte

python -m pip install requests

sudo apt install adb

## Installation de flutter et Dart

Téléchargez les fichiers compresses et extraire les dans un dossier comme indiqué dans le site puis ajoutez les binaires au path du système

Installez l’extension officielle de flutter

## Installation de Android studio et création de machine virtuelle

Téléchargez Android studio et lancez-le

Suivez l’installateur et lancez Android studio dès qu’il fini

Téléchargez SDK components

Créer un émulateur Android

Sur l'écran d'accueil d'Android Studio, sélectionnez More Actions > Virtual Device Manager

Cliquez sur Create device

Choisissez un appareil

Sélectionnez l’image système

Ajustez votre machine virtuelle puis cliquez « finish »

# Le schéma principal

# Le montage

### Les ports de Raspberry pi 3B

### Réaliser le montage suivant :

# Le code

## Le code de la carte

Le code suivant prend une image, l’envoie au serveur et attends une réponse.

Ecrire le code suivant puis exécuter le

import import requests

import RPi.GPIO as GPIO

from datetime import datetime

from time import sleep

from traceback import print_exc

import os

"""layout pins

3V3  (1) (2)  5V

GPIO GPIO2  (3) (4)  5V

B1 -->   GPIO3  (5) (6)  GND

R1 -->   GPIO4  (7) (8)  GPIO14 <--  V1

GND  (9) (10) GPIO15 <--  B2

R2 -->  GPIO17 (11) (12) GPIO18 <--  V2

B3 -->  GPIO27 (13) (14) GND

R3 -->  GPIO22 (15) (16) GPIO23 <--  V3

3V3 (17) (18) GPIO24

GPIO10 (19) (20) GND

GPIO9 (21) (22) GPIO25

GPIO11 (23) (24) GPIO8

GND (25) (26) GPIO7

GPIO0 (27) (28) GPIO1

GPIO5 (29) (30) GND

GPIO6 (31) (32) GPIO12

GPIO13 (33) (34) GND

GPIO19 (35) (36) GPIO16

GPIO26 (37) (38) GPIO20

GND (39) (40) GPIO21

"""

GPIO.setmode(GPIO.BCM)

Pins = {# B :[R, V]

5  :[7 ,8],

10 :[11 ,12],

13 :[15 ,16]

}

API_URL = "http:// :5000"

def capture_and_send_image(n):

image_path = "img.jpg"

makePic(image_path)

with open(image_path, "rb") as image_file:

file = {"image": image_file}

data = {"button": n}

response = requests.post(API_URL+"/recognize-face", files=file, data=data, timeout=500)

if response.status_code == 200:

led = Pins[n][response.json()["response"]]

GPIO.output(led, GPIO.HIGH)  # LED corr

sleep(.5)

GPIO.output(led, GPIO.LOW)

#    os.remove(image_path)

def makePic(path):

os.system("adb shell input keyevent KEYCODE_CAMERA")

sleep(.6)

os.system("adb shell input keyevent KEYCODE_CAMERA")

os.system(f"aadb pull sdcard/DCIM/camera/$(adb shell ls sdcard/DCIM/camera -t |head -1) {path}")

def button_callback(channel):

button_number = Pins.keys().index(channel)

print(f"Button {button_number} pressed")

capture_and_send_image(button_number)

for button,leds in Pins.items():

GPIO.setup(button, GPIO.IN)

GPIO.setup(leds, GPIO.OUT)

GPIO.add_event_detect(button, GPIO.RISING, callback=button_callback)

ERROR_LOG = "card.log"

with open(ERROR_LOG, mode='a') as f:

try:

while True:

sleep(.5)

except KeyboardInterrupt:

print("KeyboardInterrupt")

except Exception as e:

f.write(datetime.now().strftime("%Y%m%d%H%M%S")+"  ")

print_exc(file=f)

finally:

GPIO.cleanup()

## Le code coté serveur

Le code suivant resoit  une image et envoie une reponse, envoi une liste de l’hystorique des essais et les images correspondantes.

Ecrire le code suivant puis executer le

import json, sqlite3, os

from flask import Flask, request, jsonify, send_file

from datetime import datetime

from deepface import DeepFace

from traceback import print_exc

from flask_cors import CORS

app = Flask(__name__)

CORS(app)

# fichiers et dossiers

KNOWN_FACES_DIR = "known_faces"

PERMISSIONS_FILE = "permissions.json"

HISTORY_DIR = "history"

LOG_FILE = "attempts.db"

ERROR_LOG = "app.log"

def get_connection():

conn = sqlite3.connect(LOG_FILE)

conn.execute(

'''

CREATE TABLE IF NOT EXISTS logs (

timestamp TEXT,

button INTEGER,

response INTEGER,

person TEXT

)

'''

)

conn.commit()

conn.row_factory = sqlite3.Row

return conn

with open(PERMISSIONS_FILE) as f:

KNOWN_PERSONS = json.load(f)

# reconnaissance et log

@app.route('/recognize-face', methods=['POST'])

def recognize_face():

image = request.files['image']

button = request.form.get('button')

timestamp = datetime.now().strftime("%Y%m%d%H%M%S")

image_filename = f"{HISTORY_DIR}/{timestamp}_{button}.jpg"

image.save(image_filename)

response = False

person_name = "unknown"

try:

for person, access in KNOWN_PERSONS.items():

person_folder = f"{KNOWN_FACES_DIR}/{person}.jpg"

face_recognizer = DeepFace.verify(image_filename, person_folder)

if face_recognizer["verified"] and access[int(button)] == "1":

person_name = person

response = True

break

except ValueError:print("ValueError")

with get_connection() as conn:

log_data = (timestamp, int(button), int(response), person_name)

querry = '''

INSERT INTO logs (timestamp, button, response, person)

VALUES (?, ?, ?, ?)

'''

conn.execute(querry, log_data)

conn.commit()

return jsonify({'response': response}), 200

# envoi d'image

@app.route('/image-data', methods=['GET'])

def get_image_data():

image_filename = request.args.get('name')+".jpg"

image_path = os.path.join(HISTORY_DIR, image_filename)

if not os.path.isfile(image_path):

return jsonify({'error': 'Image not found.'}), 404

return send_file(image_path, mimetype='image/jpeg')

#liste des essais

@app.route('/attempts', methods=['GET'])

def get_attempts():

page = int(request.args.get('page', 1))

per_page = int(request.args.get('per_page', 10))

with get_connection() as conn:

cursor = conn.cursor()

query = "SELECT COUNT(*) FROM logs"

cursor.execute(query)

total_attempts = cursor.fetchone()[0]

total_pages = (total_attempts - 1) // per_page + 1

offset = (page - 1) * per_page

query = "SELECT * FROM logs ORDER BY timestamp DESC LIMIT ? OFFSET ?"

cursor.execute(query, (per_page, offset))

rows = cursor.fetchall()

return jsonify({

'attempts': [{

'timestamp': row['timestamp'],

'button': row['button'],

'response': bool(row['response']),

'person': row['person']

}

for row in rows

],

'page': page,

'has_previous_page' : page >1,

'has_next_page' : page <total_pages,

'total_pages': total_pages

}), 200

if __name__ == '__main__':

with open(ERROR_LOG, mode='a') as f:

try:

app.run(host='0.0.0.0', port=5000)

except KeyboardInterrupt:

print("Server stopped with CTRL+C")

except Exception as e:

f.write(datetime.now().strftime("%Y%m%d%H%M%S") + "  ")

print_exc(file=f)

## le code de l’application client

Le code suivant se connecte au serveur et resoit une liste de l’historique des essais et les images correspondantes.

Ecrire le code suivant puis compiler le a la platteforme desirée

import 'dart:async';

import 'package:flutter/material.dart';

import 'package:http/http.dart' as http;

import 'dart:convert';

void main() {

runApp(MyApp());

}

class GlobalVariables {

static String websiteAddress = 'localhost';

}

class MyApp extends StatefulWidget {

@override

_MyAppState createState() => _MyAppState();

}

class _MyAppState extends State<MyApp> {

bool _isDarkTheme = false;

void updateWebsiteAddress(String newAddress) {

setState(() {

GlobalVariables.websiteAddress = newAddress;

});

}

@override

Widget build(BuildContext context) {

return MaterialApp(

title: 'Image Viewer',

theme: _isDarkTheme ? ThemeData.dark() : ThemeData.light(),

home: Builder(

builder: (context) => Scaffold(

appBar: AppBar(

title: Text('log history'),

),

drawer: Drawer(

child: ListView(

padding: EdgeInsets.zero,

children: [

DrawerHeader(

decoration: BoxDecoration(

color: Colors.blue,

),

child: Text(

'Menu',

style: TextStyle(

color: Colors.white,

fontSize: 24,

),

),

),

ListTile(

leading: Icon(Icons.settings),

title: Text('Settings'),

onTap: () async {

final websiteAddress = await Navigator.push(

context,

MaterialPageRoute(

builder: (context) => SettingsPage(

currentAddress: GlobalVariables.websiteAddress,

),

),

);

if (websiteAddress != null) {

updateWebsiteAddress(websiteAddress);

}

},

),

ListTile(

leading: Icon(Icons.brightness_medium),

title: Text('Toggle Theme'),

onTap: () {

setState(() {

_isDarkTheme = !_isDarkTheme;

});

Navigator.pop(context); // Close the drawer

},

),

],

),

),

body: ImageListScreen(),

),

),

);

}

}

class ImageListScreen extends StatefulWidget {

@override

_ImageListScreenState createState() => _ImageListScreenState();

}

class _ImageListScreenState extends State<ImageListScreen> {

List<Attempt> _attempts = [];

int _currentPage = 1;

int _maxPage = 10;

bool _isLoading = false;

final GlobalKey<RefreshIndicatorState> _refreshIndicatorKey =

GlobalKey<RefreshIndicatorState>();

@override

void initState() {

super.initState();

fetchAttempts();

}

Future<void> fetchAttempts() async {

setState(() {

_isLoading = true;

});

final url = Uri.parse('http://${GlobalVariables.websiteAddress}:5000/attempts?page=$_currentPage');

final response = await http.get(url);

if (response.statusCode == 200) {

final data = jsonDecode(response.body);

final attempts = List<Attempt>.from(data['attempts'].map((x) => Attempt.fromJson(x)));

setState(() {

_attempts = attempts;

_isLoading = false;

});

} else {

setState(() {

_isLoading = false;

});

showDialog(

context: context,

builder: (ctx) => AlertDialog(

title: Text('Error'),

content: Text('Failed to fetch attempts.'),

actions: [

TextButton(

child: Text('OK'),

onPressed: () {

Navigator.of(ctx).pop();

},

),

],

),

);

}

}

Future<void> goToPage(int page) async {

setState(() {

_currentPage = page;

});

await fetchAttempts();

}

void viewImageDetails(Attempt attempt) {

Navigator.push(

context,

MaterialPageRoute(

builder: (context) => ImageDetailsScreen(attempt: attempt),

),

);

}

Widget buildPageNumber(int page) {

final bool isCurrentPage = page == _currentPage;

final bool isClickable = !isCurrentPage;

return MouseRegion(

cursor: isClickable ? SystemMouseCursors.click : SystemMouseCursors.basic,

child: GestureDetector(

onTap: isClickable ? () => goToPage(page) : null,

child: Container(

decoration: BoxDecoration(

color: isCurrentPage ? Colors.blue : Colors.transparent,

borderRadius: BorderRadius.circular(4),

),

padding: EdgeInsets.all(8),

child: Text(

'$page',

style: TextStyle(

color: isCurrentPage ? Colors.white : Colors.black,

fontWeight: isCurrentPage ? FontWeight.bold : FontWeight.normal,

),

),

),

),

);

}

List<Widget> buildPageNumbers() {

final List<Widget> pageNumbers = [];

// Add the first page number

pageNumbers.add(buildPageNumber(1));

// Add the page numbers with ellipsis in between

if (_currentPage > 3) {

pageNumbers.add(Padding(

padding: EdgeInsets.symmetric(horizontal: 4),

child: Text('...'),

));

}

for (int i = _currentPage - 1; i <= _currentPage + 1; i++) {

if (i > 1 && i < _maxPage) {

pageNumbers.add(buildPageNumber(i));

}

}

if (_currentPage < _maxPage - 2) {

pageNumbers.add(Padding(

padding: EdgeInsets.symmetric(horizontal: 4),

child: Text('...'),

));

}

// Add the last page number

pageNumbers.add(buildPageNumber(_maxPage));

return pageNumbers;

}

Future<void> _handleRefresh() async {

await fetchAttempts();

}

@override

Widget build(BuildContext context) {

return Scaffold(

body: RefreshIndicator(

key: _refreshIndicatorKey,

onRefresh: _handleRefresh,

child: _isLoading

? Center(child: CircularProgressIndicator())

: _attempts.isEmpty

? Center(child: Text('No attempts found.'))

: ListView.builder(

itemCount: _attempts.length,

itemBuilder: (ctx, index) {

final attempt = _attempts[index];

return ListTile(

title: Text(attempt.timestamp),

subtitle: Text(

'Button ${attempt.button}, ${attempt.person}, ${attempt.response ? "Allowed" : "Denied"}'),

onTap: () {

viewImageDetails(attempt);

},

);

},

),

),

bottomNavigationBar: BottomAppBar(

child: Row(

mainAxisAlignment: MainAxisAlignment.center,

children: buildPageNumbers(),

),

),

);

}

}

class ImageDetailsScreen extends StatelessWidget {

final Attempt attempt;

ImageDetailsScreen({required this.attempt});

@override

Widget build(BuildContext context) {

return Scaffold(

appBar: AppBar(

title: Text('Image Details'),

),

body: Column(

children: [

Image.network(

'http://${GlobalVariables.websiteAddress}:5000/image-data?name=${attempt.timestamp}_${attempt.button}',

loadingBuilder: (context, child, progress) {

if (progress == null) return child;

return Center(child: CircularProgressIndicator());

},

),

SizedBox(height: 16),

Text('Timestamp: ${attempt.timestamp}'),

Text('Button: ${attempt.button}'),

Text('Response: ${attempt.response ? "Allowed" : "Denied"}'),

Text('Person: ${attempt.person}'),

],

),

);

}

}

class SettingsPage extends StatefulWidget {

final String currentAddress;

SettingsPage({required this.currentAddress});

@override

_SettingsPageState createState() => _SettingsPageState();

}

class _SettingsPageState extends State<SettingsPage> {

late TextEditingController _controller;

@override

void initState() {

super.initState();

_controller = TextEditingController(text: widget.currentAddress);

}

@override

Widget build(BuildContext context) {

return Scaffold(

appBar: AppBar(

title: Text('Settings'),

),

body: Padding(

padding: const EdgeInsets.all(16.0),

child: Column(

crossAxisAlignment: CrossAxisAlignment.start,

children: [

Text(

'Website Address',

style: TextStyle(fontSize: 18, fontWeight: FontWeight.bold),

),

SizedBox(height: 8),

TextField(

controller: _controller,

decoration: InputDecoration(

border: OutlineInputBorder(),

hintText: 'Enter website address',

),

),

SizedBox(height: 16),

ElevatedButton(

onPressed: () {

Navigator.pop(context, _controller.text);

},

child: Text('Save'),

),

],

),

),

);

}

@override

void dispose() {

_controller.dispose();

super.dispose();

}

}

class Attempt {

final String timestamp;

final int button;

final bool response;

final String person;

Attempt({

required this.timestamp,

required this.button,

required this.response,

required this.person,

});

factory Attempt.fromJson(Map<String, dynamic> json) {

return Attempt(

timestamp: json['timestamp'],

button: json['button'],

response: json['response'],

person: json['person'],

);

}

}

## Capture d’écran de l’application